# AMR Features Implemented in the ESP32 Device and Server

Engineering implementation status  |  Firmware 2.0.2  |  26 October 2026

The current AMR system implements end-to-end meter acquisition, scheduled profile collection, BLE access, cellular upload, signed device messaging, persistent server storage, REST access, and a browser dashboard. The Node.js server implementation is covered by 24 automated tests, all of which pass in the current repository state.

# Implementation at a glance

| Area | Implemented capability | Current evidence |
| --- | --- | --- |
| ESP32 meter interface | DLMS COSEM acquisition over HDLC with association discovery, scaler handling, value reads, and IS 15959 profile reads | Implemented in firmware source |
| Device connectivity | EC200U packet data, TCP transfer, network time, IMEI identity, BLE GATT access, and persistent NVS settings | Implemented in firmware source |
| Data reliability | Queued readings, retained serial logs, resend on transfer failure, profile markers, and server-side deduplication | Implemented end to end |
| Security | HMAC signed messages, optional signed-only enforcement, authenticated OTA offers, and encrypted device registration | Implemented in device and server code |
| Server | TCP ingestion, validation, SQLite storage, daily logs, REST API, browser dashboard, login protection, and retention | 24 of 24 automated tests pass |

# System flow

The ESP32 reads the electricity meter over UART using DLMS COSEM and identifies values by OBIS code.

The scheduler combines due register and profile work into one meter session and creates a structured reading payload.

The EC200U modem sends queued readings and captured serial logs to the AMR data server over TCP.

The server validates the frame and signature, stores structured data in SQLite, appends logs to daily files, and exposes the data through the REST API and dashboard.

# ESP32 device features

## Meter communication and discovery

•  Communicates with the meter over UART1 using DLMS COSEM HDLC at the configured meter baud rate, client address, server address, and low-level authentication.

•  Supports DTR wake control for the external meter interface and retries meter connection initialization when the first association attempt fails.

•  Reads the meter association view once per boot, discovers supported objects and attributes, reads scaler and unit metadata, and maps known OBIS codes to readable names.

•  Reads numeric and text values from supported Data, Register, Extended Register, Demand Register, Clock, and related value classes.

•  Reads the invocation counter before connection when configured, providing a path for secured DLMS sessions even though the current build uses low authentication by default.

## IS 15959 profile acquisition

| Profile | OBIS pattern | Implemented behavior |
| --- | --- | --- |
| Instantaneous | 1.0.94.91.0.255 | Reads the current snapshot during profile cycles and uploads every row read |
| Block load | 1.0.99.1.0.255 | Uses range access when possible and uploads rows newer than the saved marker |
| Daily load | 1.0.99.2.0.255 | Runs on the first successful cycle after midnight and uploads new daily rows |
| Billing | 1.0.98.1.0.255 | Uploads the changing current row each cycle and closed months once |
| Event logs | 0.0.99.98.0 to 5.255 | Uploads new events and adds readable event names when an event code is available |
| Name plate and other profiles | Discovered at runtime | Included in a full read at boot or on a BLE read request |

•  Reads profile capture-object columns and applicable scaler profiles before converting cells to numbers, strings, timestamps, or null values.

•  Limits large catch-up reads to seven days or 400 periods and skips unsafe plain-buffer reads above 400 entries.

•  Persists per-profile newest-row markers in NVS only after the related reading is successfully transmitted.

## Scheduling and local control

•  Defaults to register reads every 15 minutes and profile reads every 30 minutes. The minimum accepted settings are 30 seconds for register reads and 5 minutes for profile reads.

•  Aligns intervals that divide a day to the meter clock, with a 60-second delay after the boundary so the meter can close the preceding block.

•  Falls back to uptime-based scheduling until the meter clock has been read, then resynchronizes without creating a tight retry loop.

•  Runs a complete read at boot and when the BLE application requests a read. Register and profile work due at the same time share one meter session.

•  Supports serial and BLE commands for status, immediate OTA check, runtime configuration, reboot, and manual profile-marker reset. BLE changes require the current device key once the device is provisioned.

# Device connectivity and delivery

## BLE interface

•  Advertises a device name derived from the Bluetooth MAC address and exposes a meter service with command, data notification, and status characteristics.

•  Accepts read, cached-value, status, register-interval, and profile-interval requests from a connected application.

•  Streams readings as newline-delimited JSON, fragments notifications to the negotiated MTU, and retains the last complete reading for later retrieval.

•  Publishes idle, reading, or error state together with result code, value count, reading age, and current schedule settings.

•  Restarts advertising automatically after a client disconnects and hosts a second Nordic UART style service for OTA administration commands.

## EC200U cellular link

•  Initializes the modem over UART2, reads the IMEI, configures the APN, attaches to the packet network, activates the PDP context, and obtains network time.

•  Opens buffered TCP sockets, transfers data in bounded chunks, checks modem SEND OK responses, detects peer closure, and reinitializes the modem after connection failures.

•  Uses the IMEI as the normal device identity, with a temporary ESP MAC based identity available before the IMEI is read.

## Reading and log delivery

•  Builds one READING payload per cycle containing the result code, numeric or text register values, and any collected profile rows.

•  Keeps up to 48 reading payloads in PSRAM or heap memory, which represents 12 hours at the default 15-minute interval. The oldest entry is dropped when the queue is full.

•  Captures normal serial trace output in a 256 KB ring buffer and uploads up to 128 KB per cycle after meter communication is idle.

•  Removes readings and log bytes only after the modem reports successful transmission. Failed transfers remain queued for the next cycle.

•  Includes boot identifiers, per-type sequence numbers, firmware version, reading age, dropped-log count, and uptime in the message headers.

•  Adds an HMAC SHA-256 signature when the device key and IMEI are available. The signature covers the message type, device identity, boot, sequence, and SHA-256 body hash.

## OTA update and registration

•  Checks an OTA service at boot, on a configured interval, or on command; the service may return a device-specific check interval.

•  Authenticates checks and reports with the device key and verifies the server's signed update offer before downloading any firmware.

•  Streams the image to the inactive OTA partition and switches boot partitions only after content length, MD5, write completion, and ESP image validation succeed.

•  Stores the pending version across reboot and reports whether the new image is running.

•  Can register automatically with a P-256 key pair. The device key is returned encrypted with a key derived from ECDH and SHA-256, then stored in NVS.

# Server features

## TCP ingestion and validation

•  Accepts multiple READING and LOG messages on one TCP connection. Each message contains a JSON header line followed by an exact byte-count body.

•  Validates message type, device identifier, unsigned integer fields, size limits, firmware field length, and optional signature format before processing the body.

•  Parses incrementally, including messages split across packets or combined in one packet. A framing error returns an error and closes the connection because the next message boundary is no longer reliable.

•  Limits connection count and idle duration and returns a structured success, duplicate, or error response for every complete message.

•  Validates reading JSON, OBIS formats, value types, profile columns, row lengths, timestamps, event names, and configured maximum counts before database insertion.

## Security controls

| Control | Implemented behavior |
| --- | --- |
| Device message verification | Derives the device key as HMAC SHA-256 of the master secret and device ID, then verifies the signed message surface with timing-safe comparison |
| Signed-only mode | REQUIRE_SIGNED can reject unsigned messages; invalid signatures are always rejected when a master secret is configured |
| Dashboard authentication | HTTP Basic authentication with constant-time credential checks; repeated failures trigger a configurable per-IP lockout |
| Local-only safe default | When no view password is configured, the HTTP service binds to 127.0.0.1 unless explicitly overridden |
| Browser hardening | Sets Content Security Policy, MIME sniffing protection, and a no-referrer policy; disables the Express identity header |

## Persistent storage and deduplication

•  Stores devices, read cycles, individual values, profile definitions, profile rows, and log-upload metadata in SQLite.

•  Deduplicates reading and log messages by device, boot identifier, and sequence number when the boot identifier is nonzero.

•  Deduplicates profile rows per device and profile. Timed rows use their meter-local timestamp as the key; event or untimed rows use a content hash. A resent timed row can replace its stored cells.

•  Computes read time from receipt time minus device-reported queue age, capped at 30 days.

•  Appends raw device traces to per-device daily log files in the configured time zone and records upload metadata including verification state and lost-byte count.

•  Optionally prunes old readings, profiles, upload metadata, and daily log files once per hour according to RETENTION_DAYS.

# Dashboard and REST access

## Browser dashboard

| View | Implemented functions |
| --- | --- |
| Devices | Searchable IMEI list, last-seen time, firmware version, signed or unsigned status, and green, amber, or red freshness indicator |
| Latest | All values from the newest read cycle with an OBIS or name filter and read metadata |
| History | A selected numeric OBIS value over 24 hours, 7 days, or 30 days as an SVG chart and table |
| Profiles | Profile selection, time-range filtering, standard tables, and a transposed view for wide billing or instantaneous data |
| Readings | Recent cycles with time, result code, value count, firmware version, and signature state |
| Log | Available log dates and the last 256 KB of the selected daily serial log |

## REST API

| Method and path | Purpose |
| --- | --- |
| GET /api/health | Unauthenticated service health response |
| GET /api/devices | Devices ordered by most recent activity |
| GET /api/devices/:id/latest | Newest reading with all stored values |
| GET /api/devices/:id/readings | Read-cycle list or one OBIS value over time with from, to, and limit filters |
| GET /api/devices/:id/profiles | Available profiles with columns, row count, and newest row time |
| GET /api/devices/:id/profiles/:obis | Profile rows with meter-local range and limit filters |
| GET /api/devices/:id/log-dates | Dates that have a stored device log |
| GET /api/devices/:id/logs | Tail of a selected daily log with truncation metadata |

## Operations and maintainability

•  Runs on Node.js 18 or newer with Express and better-sqlite3 and can be installed as a systemd service using the included unit file.

•  Loads settings from environment variables and an optional .env file, with environment variables taking precedence.

•  Includes a sample sender and a receive-check tool that can send one reading and one log and confirm both through the API.

•  Includes unit and end-to-end tests for framing, signatures, payload validation, storage, deduplication, API behavior, login lockout, compatibility, and retention.

# End to end reliability behavior

| Condition | Device behavior | Server behavior |
| --- | --- | --- |
| Normal cycle | Reads due data, queues a payload, uploads readings and trace | Validates, stores, and exposes the new data |
| Network failure | Keeps unsent readings and trace for the next cycle; modem is reinitialized | No partial message is stored |
| Lost acknowledgement or resend | May transmit the same boot and sequence again | Returns duplicate success and stores the message once |
| Power outage during profile backlog | NVS marker remains at the last successfully sent row | Repeated profile rows are deduplicated |
| Meter read failure | Queues the cycle with its nonzero result code | Stores the failed cycle so it is visible in readings |
| Invalid signature | No special retry acknowledgement handling in the current firmware | Rejects the message without storing it; the TCP connection remains usable |
| Malformed framing | Transfer fails and data remains queued when SEND OK is not obtained | Returns an error and closes the connection |
| Oversized local queue | Drops the oldest reading and rolls back profile markers so rows can be collected again | Profile and message deduplication prevents duplicate records |

# Current configuration and integration items

| Item | Current implementation | Integration note |
| --- | --- | --- |
| Firmware version | 2.0.2 | Server profile handling and dashboard profile views expect 2.0.2 data |
| Register schedule | 15 minutes by default | Runtime configurable through BLE or SET; minimum 30 seconds |
| Profile schedule | 30 minutes by default | Runtime configurable in whole minutes that divide one day; minimum 5 minutes |
| Data TCP port | Firmware build configuration uses 7001 | Server default is 5100, so TCP_PORT must be set to 7001 or the firmware build value must be changed |
| OTA service | HTTP service expected on port 3000 | The OTA server is an external companion service; this repository contains the ESP32 OTA client and AMR data server |
| Signing rollout | Unsigned devices may be accepted and marked unverified | Configure the shared master secret, approve devices, then enable signed-only mode |
| Transport encryption | AMR upload and OTA client use TCP or HTTP with application signatures | HMAC protects authenticity and integrity, but network payload confidentiality requires a trusted private network or an encrypted tunnel |
| Hardware validation | Firmware paths are implemented in source | Server behavior is automated-tested; field validation should cover the target meter, SIM, signal conditions, and OTA partition layout |